Read Me file

This e-mail contains the files necessary to re-program the flash memory of the DSP-60, and the files for the Laptop Interface Software. Copy all the files into a DSP-60 folder by themselves, and run the programs from there. Note that the files with extension “.new” must have the extension changed to “.exe”, and “.old” changed to “.bat” before trying to re-program the flash memory or run the Laptop Interface. These extensions were changed to prevent e-mail virus checkers blocking these files.

Please note that the programs can be run from a floppy, but are slow, and storage space is limited.

Re-program FLASH Memory

To re-program the flash, connect the computer serial port to the DSP-60 serial port connector (either the DB-9 connector on the circuit board, or the DB-9 connector on the back panel), using a straight through serial cable. This is the same connection used for the Interface Software.

Turn the DSP-60 power on and wait 15 seconds for the boot sequence to complete. Then navigate the computer to the DSP-60 folder and run the batch file called Program.bat. When the connection is made, the computer will open a DOS window with the message "Waiting for remote response...", which requires the operator to press the RESET button on the front edge of the bottom circuit board inside the DSP-60.

After the RESET is pressed, the DOS window will display the actions of the programming function. When the programming is complete, press the ESC key to complete the batch file run, and then the operator can close the DOS window.

After re-programming, power the DSP-60 down and back up to verify installation.

Laptop Interface

The Laptop Interface software is run using the same serial port connection described above. Run the software from the hard disk drive by starting the DSP60.EXE file. This will allow storing .dsp configuration files in the same folder, as they are saved during auditorium set-up. These files can then be downloaded, when necessary, to the DSP-60’s easily from the same place.